| The bidder: INVITATION TO TENDER | No. | 13307/2015 |
| --- | --- | --- |

| Name: |  |
| --- | --- |
| Address: |  |

| Type of purchasing: |  | Service |  | Material | x | Equipment |  | Construction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

We are kindly asking you to send us your Bid for:

| No. | Description of the equipment |
| --- | --- |
| 1 | CNC MACHINE FOR COMPUTER AIDED MANUFACTORING as per enclosed technical description and specification |

The tender documentation is available on the internet:  http://www.ijs.si/ijsw/Objave.

Queries relating to the tender documentation can only be placed through the Public-Procurement Portal of the Official Gazette of the Republic of Slovenia: http://www.enarocanje.si

The bidders can submit their bids in Slovene or English as per enclosed instructions for the bidders and technical specifications of the equipment. Variants are not permitted.

Due date for the submission of the bids is 4 March 2015 by 11.00 a.m.

| Address for the submission of the bids: | Institut “Jožef Stefan” VLOŽIŠČE Jamova cesta 39 SI-1000 Ljubljana Slovenia |
| --- | --- |

Tender documentation has to be submitted in two copies (tied together with a string and sealed in such a way that individual sheets or attachments cannot be added, taken away or replaced at a later stage) in an envelope, on which the following has to be clearly written:

- »NE ODPIRAJ – PONUDBA ZA CNC STROJ ZA RACUNALNISKO OBDELAVO MATERIALOV « - an instruction in Slovene, according to which the envelope should not be opened by the mail service as it includes a bid for CNC machine for computer aided manufactoring

- The exact address of the bidder

The bidder shall seal the bid documentation in one outer envelope (while in case of a large bulk of documentation, he shall place it into several outer envelopes, which shall be properly numbered).

The Form 7 (Envelope) should be stuck on to the outer envelope of the bid.

The public opening of the bids will be held on 4 March 2015 at 12 a.m. at the Jožef Stefan Institute, Jamova cesta 39, Ljubljana. The representatives wishing to be present at the opening, have to present a letter authorising them to attend the event to the selection board prior to the opening of the bids.

We look forward to receiving your bid.

| Ljubljana, | February 2015 |  | Jožef Stefan Institute |
| --- | --- | --- | --- |

Ljubljana

1. INSTRUCTIONS FOR THE BIDDERS

The public tender is carried out on the basis of Article 30a of the Public Procurement Act (PPA-2) (official consolidated text PPA-2-UPB5, the Official Gazette of the Republic of Slovenia, Nos.12/2013 and 19/2014) as low-value public procurement procedure.

PrEPARATION AND VALIDITY OF THE BID:

The bid has to be prepared in line with the data from the enclosed specifications. The bid has to be valid until 31 May 2015.

LANGUAGE IN WHICH THE BID SHOULD BE SUBMITTED:

The bid can be submitted in Slovene or English.

TENDER DOCUMENTATION CONSISTS OF THE FOLLOWING FORMS:

- The bid  – FORM 1

- Details about the bidder – FORM 2

- General data concerning the bidder/consortium – FORM 3

- Declaration of compliance with the conditions – FORM 4

- Statement regarding criminal offences, relating to legal entities, and the authorisation for obtaining the data from the criminal records – FORM 4.1

- Statement of the bidder’s legal representative regarding criminal offences, relating to an individual, and the authorisation for obtaining data from the criminal records – FORM 4.2

- Acting with subcontractors – FORM 5

- Sample contract (a completed, signed and stamped sample contract, initialled on each page) – FORM 6

- Envelope – OBRAZEC 7

The submitted forms have to be completed, signed and stamped. The entire documentation has to be printed out and signed by the person authorised to represent the bidder.

All costs associated with the preparation and submission of the bid shall be borne by the bidder.

ACTING AS A JOINT VENTURE OR A CONSORTIUM:

A group of economic operators are allowed to submit a bid as a joint venture or a consortium. In doing so, the group of economic operators is not required to assume a specific legal form. However, the group shall submit a Partnership Agreement, stipulating how the public contract awarded within this public-procurement procedure will be executed, specifying, in particular, the leading partner authorized to assume, from the Awarding Authority, the obligations, instructions, and possibly also the payments on behalf of, and for the account of, all the partners, as well as the share and type of services to be performed by each individual partner. The Agreement should clearly stipulate that, in relation to the Contracting Authority, all the partners are collectively and individually liable for the realisation of the entire commitment and each part of it.

Each partner in the group has to meet the competencies requirements from FORM 4.1 and FORM 4.2. On the other hand, the compliance with the other awarding authority's requirements from the above form shall be examined collectively.

When appropriate, the bidder can, in the case of individual orders, refer to the capacities of another partner within the group, irrespective of the legal nature of the bidder's relation with this partner. In such a case the bidder has to submit, to the awarding authority, a proof of having sufficient funds needed for the completion of the contract. This proof can be a written agreement made for this purpose, giving also the name of the legal entity responsible for communicating with the awarding authority.

acting with subcontractors:

In a subcontracting relationship the prime contractor transfers the execution of the whole or a part of the awarded public contract to the third party, namely a subcontractor. The definition of the subcontractor is explained in point 15a of the first paragraph of Article 2 of the Public Procurement Act (PPA-2).

A subcontractor cannot be a business entity that operates as a company associated with the main contractor in line with the law governing companies. In such a case, direct payments shall be made to the subcontractor, a legal entity or individual that is actually providing, for the company associated with the selected contractor, the goods or services required by the public contract.The selected contractor has to make a contract with this subcontractor, determining the responsibilities and the rights associated with the public contract.

The details about the subcontractor and the services to be carried out by the subcontractor, its amount, value, place and deadline are the mandatory elements of this contract, while direct payments to subcontractors are made in line with PPA-2. The bidder carrying out the contract with one or more subcontractors has to have, in line with Point 9 of Article 71 of PPA-2, contract(s) with the subcontractor(s) made at the time of making the bidder’s contract with the contracting authority or during the duration of this contract. A subcontractor has to submit a copy of such a contract made with its client (the bidder) to the contracting authority in five days after the conclusion of this contract.

In a public procurement contract, whose execution is partially or wholly transferred to subcontractors, the main contractor shall authorize the awarding authority to make direct payments to a subcontractor on the basis of an approved invoice, while the subcontractor is obliged to give its consent to direct payments. Payment terms for the main contractor and its subcontractors are the same.

When acting with subcontractors the bidder has to submit the following forms:

- The bid  – FORM 1

- Details about the bidder – FORM 2

- General data about the bidder and the consortium – FORM 3

- Declaration of compliance with the conditions – FORM 4

- Statement regarding criminal offences, relating to legal entities, and the authorisation for obtaining the data from the criminal records – FORM 4.1

- Statement of the bidder’s legal representative regarding criminal offences, relating to an individual, and the authorisation for obtaining data from the criminal records – FORM 4.2

- Partnership Agreement (only in the case the bidder acts as a party in a joint venture or a consortium of bidders)

- Acting with subcontractors – FORM 5

- Declaration of compliance with the conditions for a subcontractor – FORM 5.1

- The subcontractor’s consent to direct payments made by the contracting authority to the subcontractor(s) – FORM 5.2

- The authorisation of the bidder allowing the contracting authority to make direct payments to the subcontractor(s) on the basis of approved invoices – FORM 5.3

- The completed, signed and stamped sample contract, initialled on each page – FORM 8

The submitted forms have to be completed, signed and stamped. The entire documentation has to be printed out and signed by the person authorised to represent the bidder.

The main contractor involving one or more subcontractors has to have a contract or contracts made at the time of making its contract with the contracting authority or during the duration of this contract.

Regardless of the number of the subcontractors, only the main contractor (the bidder) is liable to the contracting authority for the obligations undertaken within this public procurement.

SUBMITTING DATA TO THE CONTRACTING AUTHORITY:

At the contracting authority’s request, the selected bidder has to submit, in line with the 13th paragraph of Article 71 of PPA-2, the following data during the tendering procedure or during the realisation of the awarded contract:

- data about its founders and partners including dormant and limited partners, shareholders and other owners, together with the ownership shares of the above entities;

- data about the business entities treated as the companies associated with the main contractor in line with the law governing companies.

The selected bidder has to submit the above data in 8 days after the receipt of the awarding authority’s request.

If the State in which the Candidate or Bidder has its seat does not issue such documents, he may submit a sworn witness statement or a sworn Candidate or Bidder statement. Except in the case of tender collection procedure, the Statement shall be given before judicial or administrative authority, notary, or a competent vocational or  commercial body in the State where the Candidate or Bidder has its seat.

selection of the most favoUrable bidder:

When assessing the bids, the awarding authority’s selection board shall select the most favourable bidder among the timely submitted bids on the basis of the bidding price, provided that all technical requirements are fulfilled (see section 2). Otherwise the bid will be rejected.

LEGAL PROTECTION:

Legal protection for bidders, client and public interest in the process of awarding the contract is regulated by the Legal Protection in Public Procurement Procedures Act (the Official Gazette of the Republic of Slovenia, No. 43/2011, 63/2013 ZPVPJN).

A request for revision, relating to the content of the publication and / or tender documentation, except in the case of the fourth paragraph of Article 25 of the ZPVPJN, shall be filed within eight working days from the date of publication of the contract notice or the notice of the additional information, information on incomplete procedure or correction, if this notice amend or supplement the requirements or criteria for selecting the best bidder in the tender documentation or previously published information about the contract, but not after the deadline for receipt of the tenders.

The applicant must pay a tax of 1.500,00 euros to the bank account of the Ministry of Finance, IBAN no. SI56 0110 0100 0358 802, name of the bank is the Bank of Slovenia, Slovenska 35, 1505 Ljubljana, Slovenia, SWIFT CODE: BS LJ SI 2X – a tax for revision of the public-procurement procedure.

A request for revision shall be filed in writing directly to the contracting authority, sent by post as registered mail, or sent off as registered mail with an acknowledgment of receipt or electronically, in line with the first paragraph of Article 24 of the ZPVPJN. At the same time the applicant for revision shall send a copy of the revision request also to the ministry responsible for finance.

| Ljubljana, | February 2015 |  | Jožef Stefan Institute |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ljubljana |  |

2. TECHNICAL DESCRIPTION AND SPECIFICATIONS OF THE EQUIPMENT

The subject of public tender is CNC computer aided manufacturing machine. The machine will be used for protopyping and manufacturing of mechanical detector parts that will be typically used in detectors of large international collaborations like ATLAS at CERN and Belle at KEK.

TECHNICAL CHARACTERISTICS:

- Repeat accuracy ± 0.02 (mm)

- automatic tool change stations

- cooling air nozzle with hose

- control cabinet 230V/16A

- travel area X/Y = 1000mm/650mm

- all axes EC 60 48V/156W

- control pc with keyboard and screen

- Z-passage: 350mm  or more

- Z-traverse path: 250mm  or more

- spindle pitch X/Y/Z-axis: 16x5mm

- table clamping area X/Y-area: 1200mmx700mm

- spindle motor 2200W, 20000rpm

OTHER REQUIREMENTS:

- The equipment should be CE compatible.

- Transport should be included.

- Documentation and manuals should be included.

FORM 1

3. BID

Details about the bidder:

| Name: |  |  |  |
| --- | --- | --- | --- |
| Address: |  |  |  |
| VAT identification number: |  |  |  |
| Bank account and the name of the bank: |  |  |  |
| Contact person: |  |  |  |
| Contact person's E-mail address: |  |  |  |
| Person responsible for signing the contract: podpis pogodbe: |  |  |  |

JOŽEF STEFAN INSTITUTE

Jamova cesta 39

1000 Ljubljana

| BID No.. Place:. Date:. |
| --- |
| On the basis of the invitation to tender No. 13307/2014 we are pleased to submit the following bid: |  |  |  |  |  |  |  |  |  |
| Type of purchasing: |  | Service |  | Material | x | Equipment |  | Construction |  |

| No. | Description of the goods | Quantity | Price | Discount % | Value |
| --- | --- | --- | --- | --- | --- |
| 1 | CNC MACHINE FOR COMPUTER AIDED MANUFACTORING | 1 Set |  |  |  |

|  |  |  | TOTAL |  |
| --- | --- | --- | --- | --- |
|  |  |  | DISCOUNT |  |
|  |  |  | VALUE without VAT |  |

| Delivery time (in weeks): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Payment terms: |  | - 50% payment in advance after receipt of the order, payable on receipt of proforma invoice - 20% payment payable on receipt of final invoice - 30% payment after technical acceptance |  |  |  |
| Delivery term: | DAP Ljubljana (price includes delivery to the client's address) |  |  |  |  |
| Warranty period (at least 12 months): |  |  |  |  |  |
| Validity of the bid: |  |  | 31 May 2015 |  |  |

MANDATORY ENCLOSURE:

Brochures including a complete technical description and specification of the equipment, and the supplier’s Offer/Quotation with technical specifications and a list of components included.

|  |  | The bidder: |
| --- | --- | --- |
|  |  | (signature of the representative) (name and surname) |

FORM 2

4. DETAILS ABOUT THE BIDDER

Details about the bidder:

| Name of the bidder: |  |
| --- | --- |
| Address of the bidder: |  |
| Legal representative of the bidder: |  |
| Contact person: |  |
| E-mail of the contact person: |  |
| Telephone number: |  |
| Fax number: |  |
| Website: |  |
| VAT identification number: |  |
| Bank account and the name of the bank: |  |
| Number of the entry in the register of companies: |  |
| Person responsible for signing the contract: |  |

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

FORM  3

4.1  GENERAL DATA CONCERNING THE BIDDER/CONSORTIUM

Acting as a bidder we are submitting:

|  | a) the bid individually, as the sole contract partner |
| --- | --- |
|  | b) a joint bid with the following contract partners: |

| No. | Contract partner | Description of the work | Value | Share in % |
| --- | --- | --- | --- | --- |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| 3. |  |  |  |  |
| 4. |  |  |  |  |
| 5. |  |  |  |  |

Enclosures:

- Partnership Agreement

- Requested details about all the partners of the consortium  (Form 2, Form 4)

This statement is an integral part of, and an annex to, the bidder’s application relating to the public tender for the supply of the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING.

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

The contracting authority’s note: The bidder checks a) or b) box. If the bidder checks b) box, it should also fill-in

the table for each partner.

The contracting authority’s note: Form must be completed and attached to the bid, regardless the fact if you submit bid individually or with a joint bid.

FORM  4

4.2 DECLARATION OF COMPLIANCE WITH THE CONDITIONS

The bidder:

| Name: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Address: |  |  |  |  |  |
| VAT identification number: |  |  |  | Registration Nr.: |  |
| Name of the Register Court: |  |  |  |  |  |

Acting as a bidder, we declare that:

- we accept all the requirements from the tender documentation;

- we are licensed to provide the service that is the subject of this tender;

- neither the law nor any other regulation forbids us to make a contract, whose subject is the service tendered with this public procedure;

- on the day that is deadline for submitting a bid, we are not listed in the register of bidders with negative references from Article 77a of the Public Procurement Act (PPA-2);

- we have never been convicted by a final judgement in any country for an offence arising from our professional misconduct;

- we have never committed any serious breach of the professional code;

- we are not subject to a compulsory settlement, bankruptcy or forced termination, or any other similar procedure that would lead to the closure of our activities;

- when providing information in line with the provisions of Articles 41 to 49 of PPA-2, within this or any other previous procedure, we never intentionally gave misleading information nor failed to give the required information;

- we do not have 50 EUR or more overdue, unpaid obligations regarding the payment of social security contributions or the payment of taxes in accordance with the statuary provisions of the country where the Bidder has its head office or the provisions of the country of the Contracting Authority, settled on the day of submission the bid;

- we have paid and fulfilled all our obligations due to our subcontractors in previous public procurement procedures;

- all the details included in our bid are correct and that the attached photocopies correspond to the originals;

- we assure technical support by our qualified personnel;

and are giving the following

C O N S E N T

The undersigned agrees that the Jožef Stefan Institute may, as part of the awarding procedure, obtain the data from the official records needed for checking the fulfilment of the requirements from the tender documentation relating to the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING .

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

FORM 4.1

STATEMENT REGARDING CRIMINAL OFFENCES FROM THE 1st PARAGRAPH OF ARTICLE 42 OF PPA-2, RELATING TO LEGAL ENTITIES, AND THE AUTHORISATION FOR OBTAINING THE DATA FROM THE CRIMINAL RECORDS

Under criminal and material accountability, we hereby declare that our company has never been convicted of the following criminal offences determined in the 1st paragraph of Article 42 of PPA-2.

We also give the following

A U T H O R I S A T I O N

The undersigned authorises the Jožef Stefan Institute to obtain a statement from the Ministry of Justice relating to the criminal-record data for the purpose of checking the fulfilment of the requirements of the public tender for the supply of the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING .

| Full name of the company: |  |
| --- | --- |
| Address of the company: |  |
| Municipality where the company is established: |  |
| Entry number from the register of companies: |  |

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

FORM 4.2

STATEMENT OF THE LEGAL REPRESENTATIVE REGARDING CRIMINAL OFFENCES FROM THE 1st AND 2nd PARAGRAPHS OF ARTICLE 42 OF PPA-2, RELATING TO AN INDIVIDUAL, AND THE AUTHORISATION FOR OBTAINING THE DATA FROM THE CRIMINAL RECORDS

Under criminal and material accountability, I hereby declare that I have never been convicted of any of the following criminal offences, determined in the 1st paragraph of Article 42 of PPA-2.

I also give the following

A U T H O R I S A T I O N

The undersigned                                                     (name and surname) authorises the Jožef Stefan Institute to obtain a statement from the Ministry of Justice relating to the criminal-record data on natural persons for the purpose of checking the fulfilment of the requirements of the public tender for the supply of the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING.

| My personal details are as follows: |  |
| --- | --- |
| Date of birth: |  |
| Place of birth: |  |
| Municipality of birth: |  |
| Country of birth: |  |
| Permanent/temporary address: |  |
| Nationality: |  |
| My previous surname was: |  |

|  |  | Signature of the legal representative/agent: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

The awarding authority’s note: This form has to be completed by each legal representative of the bidder, and, for

this reason, it can be appropriately photocopied.

FORM 5

5. ACTING WITH SUBCONTRACTORS

Acting as a bidder we are submitting the bid:

|  | a) individually, without subcontractors |
| --- | --- |
|  | b) with the following subcontractors: |

| Nr. | Subcontractor | Description of the work | Value | Share in % |
| --- | --- | --- | --- | --- |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| 3. |  |  |  |  |
| 4. |  |  |  |  |
| 5. |  |  |  |  |

Enclosures: requested proofs for all partners (Form 5.1 and Form 5.2 – for the subcontractors,

Form 5.3 – for the main contractor)

This statement is an integral part of, and an annex to, the bidder’s application relating to the public tender for the supply of the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING.

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

The contracting authority’s note: The bidder checks a) or b) box. If the bidder checks b) box it should also fill in

the table for each partner.

The contracting authority’s note: Form must be completed and attached to the bid, regardless the fact if you submit bid individually or with a joint bid.

FORM  5.1

5.1 DECLARATION OF COMPLIANCE WITH THE CONDITIONS

FOR A SUBCONTRACTOR

The subcontractor:

| Name: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Address: |  |  |  |  |  |
| VAT identification number: |  |  |  | Registration No: |  |
| Name of the Register Court: |  |  |  |  |  |

We hereby declare that we comply with the following conditions:

| 1. | We are not subject to a compulsory settlement, bankruptcy or forced termination, or any other similar procedure that would lead to the closure of our activities. |
| --- | --- |
| 2. | We do not have 50 EUR or more overdue, unpaid obligations regarding the payment of social security contributions or the payment of taxes in accordance with the statuary provisions of the country where the Bidder has its head office or the provisions of the country of the Contracting Authority, settled on the day of submission the bid. |
| 3. | We accept all the requirements from the tender documentation. |

and are giving the following

C O N S E N T

The undersigned agrees that the Jožef Stefan Institute may, as part of the awarding procedure, obtain the data from the official records needed for checking the fulfilment of the requirements from the tender documentation relating to the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING.

|  |  | The subcontractor: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

The contracting authority’s note: The bidder, who does not act with subcontractors, does not need to submit this

form.

FORM 5.2

  - CONSENT OF THE SUBCONTRACTOR

C O N S E N T

of the subcontractor to direct payment

| Subcontractor |  |
| --- | --- |

We give consent to the Jožef Stefan Institute, the Contracting Authority, to make direct payments to us based on the invoices presented to the Contracting Authority by the main contractor.

This statement is an integral part of, and an annex to, the bidder’s application relating to the public tender for the supply of the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING.

|  |  | The subcontractor: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

The contracting authority’s note: The bidder, who does not act with subcontractors, does not need to submit this

form.

FORM 5.3

  - AUTHORISATION OF THE BIDDER

A U T H O R I S A T I O N

We authorise the Contracting Authority to make direct payments to the subcontractor, based on the invoices presented to the Contracting Authority by the main contractor.

This statement is an integral part of, and an annex to, the bidder’s application relating to the public tender for the supply of the CNC MACHINE FOR COMPUTER AIDED MANUFACTORING.

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

The contracting authority’s note: The bidder, who does not act with subcontractors, does not need to submit this

form.

FORM 6

SAMPLE CONTRACT

Jožef Stefan Institute, Jamova cesta 39, 1000 Ljubljana, Slovenia, represented by the Director, prof. dr. Jadran Lenarčič, henceforth referred to as “the Client”, and

………………………………………………………………….., represented by the Director, ………….,

henceforth referred to as “the Supplier”,

have agreed on the following

C O N T R A C T

Article 1

SUBJECT OF THE CONTRACT

The contracting parties have established that, in line with the Public Procurement Act, the Client awarded the contract to the most cost-effective bidder with regard to the realisation of the supply requirement expressed in the public tender published on the Public Procurement Portal of the Republic of Slovenia under the reference number NMV847/2015.

With this contract the Client and the Supplier agree on their mutual rights and obligations, necessary for the smooth operation of purchasing and selling the following device:

CNC MACHINE FOR COMPUTER AIDED MANUFACTORING

specified, in detail, in the Quotation/pro-forma invoice with the ref. number ………..... as of ………….... that is an integral part of this contract.

Article 2

VALUE OF THE CONTRACT

The value of the above equipment is …….………….. euros. The price does not include the VAT.

Article 3

DELIVERY TERMS

The delivery period for the equipment that is the subject of this contract is ………......... after the date of the contract.

The Supplier shall provide, without additional cost to the Client, transportation including insurance coverage at the Client’s address. The Supplier shall notify the Client of the intended delivery by fax, e-mail, in writing or by telephone, at least 3 working days prior to the delivery.

The Client shall sign the handover note after the equipment has been correctly delivered.

The Supplier shall provide the delivery of the equipment with his own professionally qualified staff or with the subcontractors, listed in tender documentation. If the Client finds that the work is carried out by the subcontractor not listed in the tender, he is entitled to cancel the contract. The Supplier reserves the right to verify any time subcontractor employees performing work. All employees are required to give credible information.

Article 4

PAYMENT TERMS

The Supplier shall issue the invoices to the Client in the following order:

- 50% payment in advance after receipt of the order, payable on receipt of invoice

- 20% payment payable on receipt of invoice

- 30% payment after technical acceptance

The Client is obliged to pay an invoice for the goods, i.e., the equipment, within 30 days of the correct issuing of the invoice.

The contracting parties agree that, in the case of a late payment, the Client has to pay the default interest.

The Supplier has to enclose, to its own invoice, any invoices issued by the subcontractor(s) that the Supplier had previously approved of. By signing the contract, the Supplier authorises the Client to make direct payments to the subcontractor(s) on the basis of approved invoices and an appropriately concluded contract of assignment. When acting together with one or more subcontractors, the Supplier shall make a contract or contracts with the subcontractor(s) specifying, in detail, the type and scope of the activities carried out by the subcontractor(s) and the price(s) charged for the provision of the concerned goods or services. The Suppler is fully liable for the realisation of the tasks carried out by the subcontractor(s), as if the supplier had carried them out by itself, and the client’s approval of the subcontractor(s) does not reduce the Supplier’s responsibility for an efficient and timely realisation of the contractual services relating to the supply of the equipment. At the same time the subcontractor(s) also agree(s) that the Client makes direct payments to the Subcontractor(s).

If, after the conclusion of this public contract, the Supplier replaces its subcontractor or makes a contract with another subcontractor, the Supplier shall submit the following documents to the Client within 5 days of this change:

- a written statement that all uncontested obligations to the previous subcontractor have been settled;

- a written authorisation to the Client to make direct payments to the new subcontractor for the services provided;

- a written consent of the new subcontractor to direct payments.

Article 5

ACCEPTANCE

The acceptance shall be performed on the basis of the Final acceptance certificate (Appendix 1), signed by the authorised representatives of both contracting parties, upon a proper delivery of equipment in adequate quality and quantity.

The Supplier will submit to the Client the following documentation:

- Invoice

- Delivery note (with an exact list of delivered parts, data about the gross and net weight)

- Technical documentation and instructions for use

- Precise installation instruction

- Handover note – final acceptance certificate

- Warranty certificate

It is preferable for the documentation to be in Slovene or English.

Article 6

SUPPLIER GUARANTEES

The responsibility for keeping the equipment undamaged is transferred from the Supplier to the Client once the equipment is delivered to the Client.

The Supplier guarantees to the Client:

- that the equipment is new, not used and free of any actual and legal defects;

- that the supplied equipment functions perfectly and that it complies fully with all technical descriptions, characteristics and specifications, provided by the Client;

- that the Client shall acquire all the rights related to the equipment, while the Supplier shall duly perform all the obligations related to the equipment.

The Supplier provides the Client with a warrantee to cover the quality of the equipment and its smooth operation at least to an extent equal to the extent of the warrantee given by the producer of the equipment to the Supplier, i.e., ……...years following the date of the receipt of the equipment.

During the warrantee period the Supplier shall provide for emergency services responding to the Client by E-mail or telephone within one working day after the receipt of a notification. If an on-site repair service is needed, the Supplier shall provide it within 15 working days after the problem has been identified.

If the equipment has not been repaired within the above period, the Supplier shall provide, for the time during which the equipment is being repaired, an equivalent substitute piece of the equipment or a part of the equipment. In this case the warranty period shall be extended by a time equal to the duration of the repair work.

All transport costs and other costs relating to the repair work made during the warranty period are charged to the Supplier.

Article 7

ANTI-CORRUPTION CLAUSE

If it is found that, during the execution of the public tendering, on the basis of which this contract has been made, or during the execution of this contract, anyone acting in the name of – or on the account of – the Supplier, has promised, offered or given any undue advantage to the Client (the contracting authority), or those employees that had a decisive influence on the choice of the Supplier, in order:

  - to win this public contract,

  - to conclude this contract under more favourable conditions,

  - to omit due diligence in the supervision of the contract implementation,

  - to carry out any other act or omission causing damage to the Client or allowing the Client or its employees to obtain an undue advantage,

this contract will be void.

In the case of an alleged existence of the conditions from the first paragraph of this article or from a notification of the Commission for the Prevention of Corruption, or any other responsible body, relating to their alleged existence, the Client shall initiate the proceedings for invalidity of the contract from the first paragraph of this article, or any other measures, in line with the regulations of the Republic of Slovenia.

Article 8

FINAL PROVISIONS

The contracting parties shall determine all the amendments to this contract, after a previously reached agreement, with the annexes to this contract.

The contracting parties shall resolve any disagreement relating to the realisation of this contract by common consent.

In the case that the contracting parties cannot settle a dispute by common consent, they shall abide by the decision of the competent court in Ljubljana.

This contract comes into force on the day when it is signed by both contracting parties.

The contract is drawn up in four identical copies so that each contracting party receives two copies.

| CLIENT: |  | SUPPLIER |
| --- | --- | --- |
| Jožef Stefan Institute |  |  |
| Director: |  | Director: |
| Prof. Dr. Jadran Lenarčič |  |  |
| Date: |  | Date: |

We herewith declare that the format of this sample contract is acceptable for the bidder.

|  |  | The bidder: |
| --- | --- | --- |
| (place, date) | (stamp) | (signature of the representative) |

FORM 7

ENVELOPE

(fille-in the sender and stuck on to the outer envelope of the bid )

| Sender: |  |
| --- | --- |
| Recipient: Institut »Jožef Stefan« vložišče Jamova cesta 39 SI-1000 Ljubljana SLOVENIA Phone: +386-1-477-3210 Fax: +386-1-477-3189 |  |
| NE ODPIRAJ, PONUDBA! CNC STROJ ZA RAČUNALNIŠKO OBDELAVO MATERIALOV. (Public procurement titje in Slovene). NMV847/2015. (številka objave na Portalu RS oziroma EU. |
| (Date and time – Datum in ura) | (Receiving order – Zaporedna številka predložitve) |

appendix 1: Prevzemni zapisnik za

»CNC MACHINE FOR COMPUTER AIDED MANUFACTORING«

| Name: |  |
| --- | --- |
| Address: |  |
| VAT identification number: |  |
| Contact person's: |  |
| Contact person's E-mail address: |  |
| Person responsible for signing the contract: |  |
| Contract No.: |  |

| Following Delivery note Nr. |  |  | of |  | herewith we issue : |
| --- | --- | --- | --- | --- | --- |

| FINAL ACCEPTANCE CERTIFICATE |  | * Nr. |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | place: |  |  |  |  |
|  | date: |  |  |  |  |

| Type of purchasing: |  |  |  | Service |  |  |  | Material | X | Equipment |  |  |  |  | Construction |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Delivery method: | x |  | Supplier |  |  |  | JSI arrangement: |  |  |  |  | Other: |  |  |  |  |

| Nr. | Findings |  | YES |  | NO |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | All the ordered parts or services have been delivered. |  |  |  |  |  |
| 2 | The quality of the supplied parts or services complies with all the terms of the purchase agreement. |  |  |  |  |  |
| 3 | The required quantity of individual components or services has been supplied. |  |  |  |  |  |
| 4 | All the required or necessary documentation is included. |  |  |  |  |  |
| 5 | All the manuals including the instruction for use are included. |  |  |  |  |  |
| 6 | All the other required acceptance tests have been carried out (QA) |  |  |  |  |  |
| According to the above findings, we confirm acceptance |  |  |  |  |  |  |

Additional comments:

Supplier shall receive a copy of the signed final acceptance certificate.

| The Supplier: |  | Institut “Jožef Stefan” |  |
| --- | --- | --- | --- |
| (name and surname of the supplie’s representativer) |  | (name and surname of the responsible for the equipment) |  |